# Terms of Reference (ToR)

Section 3: Scope of Work

Source: BTRC EOI Reference 14.32.0000.000.400.07.0021.25.1685

# 3. Scope of Work

The consulting/implementing firm ("the Vendor") shall perform, at minimum, the following tasks.

## 3.1 Inception, Requirement Study and System Design

Conduct detailed stakeholder consultations with:

– BTRC technical and relevant divisions.

– Selected fixed broadband service providers.

Prepare:

– Detailed Business Process Study and Gap Analysis.

– Software Requirements Specification (SRS) covering functional and non-functional requirements.

– High Level Design (HLD) and Low Level Design (LLD) including system architecture, modules, interfaces, security and data models.

Define:

– Data collection frequency, granularity (per link, per region, per package, etc.).

– KPI definitions and computation logic.

– SLAs and performance targets for the system.

## 3.2 Data Acquisition and Integration Layer

The platform must include a robust data acquisition layer capable of integrating multiple data sources:

### 3.2.1 Network Operation Data via SNMP

Service Providers shall facilitate SNMP-based metric collection from within the service provider's network infrastructure. This architecture is mandated for the following information security considerations:

Attack Surface Minimization: SNMP management planes shall not be exposed to external or public networks

Credential Protection: SNMP authentication credentials remain within ISP security perimeter

Infrastructure Isolation: Critical network management interfaces are protected from external threat actors

Only aggregated, sanitized performance data shall be transmitted to BTRC QoS Platform via secured API endpoints.

Data Collection Architecture:

Data collection shall be performed using software-based agents (containerized/Docker or equivalent) deployed within ISP infrastructure. Vendors may propose supplementary approaches for specific use cases.

Deployment Locations (Vendor to Propose):

Vendors shall propose their software agent deployment strategy. Deployment options include:

| Deployment Location | Purpose | Agent Function |
| --- | --- | --- |
| ISP PoP (Point of Presence) | Active QoS measurements | Synthetic testing: latency, throughput, packet loss, DNS, HTTP response |
| ISP NOC (Network Operations Center) | SNMP data aggregation | Collection from network devices: core routers, aggregation, BRAS |

Deployment Flexibility:

Vendors may propose unified agents serving both QoS and SNMP functions, OR specialized agents per location

Deployment topology to be detailed in vendor's technical proposal

ISPs shall provide compute infrastructure for hosting software agents

Security Requirements:

SNMP implementations must use secure protocols; systems with known security vulnerabilities are NOT permitted

Agent-to-platform communication via secure APIs (TLS encrypted)

Credentials and sensitive data must remain within ISP security perimeter

Data Collection Requirements:

Integration with routers and relevant network elements (e.g. aggregation, core, BRAS) using SNMP

Device Scope: Core routers, aggregation devices, and BRAS only; CPE (Customer Premises Equipment) devices are excluded from SNMP monitoring

Collection of real-time / scheduled network counters, including:

– Interface bandwidth utilization (upstream/downstream)

– Interface status and errors

– Broadband remote access server metrics

Support for multiple vendors' devices and MIBs (standard + vendor-specific)

Configurable polling intervals and threshold-based alerts

Automated QoS Measurement Scope:

The solution must enable automated measurement of:

Download and upload throughput

Network latency and jitter

Network trace report to multiple destinations

Packet loss

DNS resolution performance

HTTP/web response times

Website reachability (national and international)

Independent, automated measurement from ISP network points provides verifiable QoS data that is not dependent on end-user participation or ISP self-reporting. This serves as the primary source for QoS measurement and assessment as per QoS guidelines.

Vendors should describe their proposed approach for:

Automated measurement methodology

Software agent placement strategy

Data integrity and tamper-resistance

### 3.2.2 Integration with Service Provider Systems via APIs

Data Submission Model:

Data Source: Service Providers (ISPs)

Submission Method: REST API endpoints provided by QoS Platform

Data Flow: ISP Systems to BTRC REST API to QoS Platform

Responsibility: Service Providers are accountable for data accuracy, completeness, and timely submission

Service Providers shall submit data via REST API (REST/JSON or equivalent) from their internal systems including:

ERP

NMS

RADIUS server

Billing software

CRM or customer management system

Data to be collected shall include:

Operational & Product Data:

Package definitions: name, advertised speeds, pricing, data cap, FUP details (active and discontinued)

Total subscribers by Package and location

Newly added subscribers by Package and location

Removed/discontinued subscribers by Package and location

Total subscribed bandwidth per location/POP and type (e.g. Cache internet etc)

Access nodes/POPs count by district/upazila/thana

Upstream bandwidth capacity (total contracted/provisioned bandwidth per location)

Service Quality Data:

Network incident reports (outages, degradations) via web form with location data

Affected areas and restoration times

Customer complaint volumes and resolution metrics

QoS Data (where available from provider systems):

Latency, packet loss, throughput statistics, uptime

Submission Frequency: To be proposed by vendor (real-time to monthly depending on data type)

Vendor shall:

Define standard data schemas and API specifications

Implement necessary middleware, ETL and validation mechanisms

Ensure data integrity, deduplication and time-stamping

### 3.2.3 Integration with BTRC and Other Systems (if required)

Provide APIs / data feeds for:

– BTRC internal reporting systems

– Data warehouse / BI tools

Design the system such that new integrations (additional service providers, new systems) can be onboarded with minimal changes.

### 3.2.4 Service Provider Facilitation and Accountability

For ISP-Submitted Operational Data:

Service Providers are accountable for:

Data Accuracy: Ensuring submitted data accurately reflects operational status

Data Completeness: Submitting all required data categories per defined specifications

Timely Submission: Adhering to submission schedules as applicable

For Automated SNMP-Based Collection:

Service Providers are accountable for:

Facilitation: Enabling secure access for BTRC data collection within their internal network

Data Access Accuracy: Ensuring collectors can access accurate, unmodified data from network elements

Security Cooperation: Maintaining the security perimeter where SNMP credentials remain protected

Infrastructure Provision: Providing compute resources for hosting software agents within their network

### 3.2.5 Legacy DIS Data Migration

BTRC currently operates a Data Information System (DIS) that stores historical ISP operational and service data. This data must be migrated to the new QoS Monitoring Platform.

Migration Requirements:

| Aspect | Requirement |
| --- | --- |
| Data Scope | All data categories including operation, package, user data, etc. |
| Estimated Volume | Less than 2 TB |
| Database Platform | To be disclosed during RFP stage |
| Transition Strategy | Full migration with data validation prior to cutover |
| Parallel Operation | Not required; cutover approach approved |

Vendor Responsibilities:

Conduct data assessment and mapping during inception phase

Develop ETL (Extract-Transform-Load) pipelines for data migration

Perform data validation and integrity verification

Execute migration with minimal service disruption

Provide migration completion report with data reconciliation

Migration Deliverables:

Data migration plan and mapping document

ETL scripts and tools

Validation and reconciliation report

Post-migration data reconciliation report

## 3.3 Operational Data Module

This module shall provide:

Real-time / near real-time dashboards on:

Total upstream bandwidth and utilization by:

– Provider

– Region (district/upazila/thana or other defined hierarchy)

– Time period

Number of users:

– Total active users.

– New activations / churn (if data available).

Coverage:

– Map-based coverage visualization.

– Number of access nodes / POPs per region.

Historical trends and comparative views across:

Different service providers.

Geographic regions.

Time periods (hourly/daily/monthly).

## 3.4 QoS Monitoring Module

The QoS monitoring module must compute, display and store (for analytics and audit):

Key QoS parameters:

| Parameter | Description |
| --- | --- |
| Service Availability | Network uptime percentage |
| Download Speed | Actual vs. advertised speed ratio |
| Upload Speed | Actual vs. advertised speed ratio |
| Network Latency | Round-trip delay (in-country and international), including 95th percentile |
| Packet Loss | Data transmission reliability |
| Jitter | Latency consistency |
| DNS Performance | Name resolution speed |
| Web Page Reachability | Web page reachability status |
| Web Site Response Time | Web site response time in ms/s |
| Network Trace | Trace route to national and international IP targets for path check |
| Service Degradation Detection | Automatic identification of QoS falling below thresholds |
| Outage Detection | Automatic detection and visualization of major incidents |

Note: Specific benchmarks and thresholds will be defined in consultation with stakeholders and referenced against international standards (e.g., ITU, APT).

Features:

Configurable QoS thresholds for each parameter and provider.

Automatic detection and visualization of:

– Service degradations.

– Major incidents and outages.

Geo-spatial view to detect regional QoS issues.

Drill-down capabilities from national to operator to region to specific network segment.

KPI computation engine aligned with BTRC QoS regulations/definitions.

QoS data shall be derived from:

Network element counters (via SNMP collection facilitated within ISP internal networks, transmitted via secured API).

Provider NMS / performance systems (via ISP data submission to REST API).

User-end measurement app data (crowd-sourced, see section 3.6).

## 3.5 Revenue and Package Analytics Module

This module will use data obtained from service providers' ERP/billing/CRM systems via APIs and shall provide:

Revenue Metrics:

Total revenue per provider, per period.

Revenue by package, region, and customer segment (where data available).

ARPU and related indicators (if derivable from provided data).

Package & Tariff Metrics:

Master list of all active and historical broadband packages.

Tariff of each package and associated attributes (speed, data cap, FUP, etc. as available).

Number of users per package.

Package-wise QoS and churn analysis (where data supports such mapping).

Analytics and Reporting:

Time series trends.

Comparative views across operators and regions.

Export to Excel/PDF/CSV.

## 3.6 User-facing Application (Mobile and/or Web)

The Vendor shall design, develop and deploy a user-friendly app (Android mandatory; iOS and/or web interface preferable) with at least the following functionalities:

Speed Test:

Download and upload speed measurement.

Logging results with timestamp, location (with user consent), and operator/package info (where possible).

Jitter Measurement:

Measurement of jitter and, where relevant, packet loss.

Webpage Loading Time:

Test of page load time for predefined popular or reference websites.

Display results to user and simultaneously send anonymized/aggregated data to central platform.

Additional (to be confirmed by BTRC):

Display of historical test results to user.

Optional complaint/feedback submission linked to test results.

Language support: Bengali and English.

Required Data Types from End-User Application:

Speed test results (download/upload)

Network quality metrics (latency, jitter, packet loss)

DNS and webpage response times

Geographic location (with user consent)

ISP and connection identification

Timestamp and device information

Service Provider/Package/Tariff details (where possible)

Complaint/feedback submission linked to specific test results (optional)

Submission: As and when user submits

The app must:

Follow best practices for performance, UX and security.

Obtain necessary permissions and user consent for data collection.

Send measurement data securely to the central platform via APIs.

## 3.7 Regulatory/Monitoring Tool

The Vendor shall design, develop and deploy a regulatory tool/ app (Android mandatory; iOS and/or web interface preferable) with at least the following functionalities:

Speed Test:

Download and upload speed measurement.

Logging results with timestamp, location and operator/package info (where possible).

Jitter Measurement:

Measurement of jitter and, where relevant, packet loss.

Webpage Loading Time:

Test of page load time for predefined popular or reference websites.

Display results to user and simultaneously send anonymized/aggregated data to central platform.

Additional (to be confirmed by BTRC):

Display of historical test results to user.

Language support: Bengali and English.

Required Data Types:

Speed test results (download/upload)

Network quality metrics (latency, jitter, packet loss)

DNS and webpage response times

Geographic location (with user consent)

ISP and connection identification

Timestamp and device information

Service Provider/Package/Tariff details (where possible)

## 3.8 Data Storage, Analytics and Dashboard

Centralized database(s) designed for:

– High volume time-series data.

– Historical storage with configurable retention policies.

Analytics engine and dashboards for:

– BTRC management.

– Technical users/analysts.

Features:

– Role-based dashboards and access control.

– Custom report builder with filters (provider, region, time, KPI).

– Scheduling of periodic reports (e.g. monthly QoS summary).

Platform Capability Requirements:

| Capability | Requirement |
| --- | --- |
| Scalability | Handle data from 1500+ ISPs and millions of measurements |
| Availability | High uptime with disaster recovery provisions |
| Data Retention | 1 year storage for historical analysis and audit |
| Security | End-to-end encryption, access controls, audit trails |
| Integration | Standard interfaces for ISP system connectivity |
| Drill-down Analytics | Navigation from national to operator to region to network segment level |
| Map-based Visualization | Geographic/geo-spatial coverage and QoS visualization |
| Time Granularity Options | Data views by hourly, daily, weekly, monthly periods |
| Comparative Analytics | Cross-provider and cross-region comparison views |
| Data Polling Resolution | Maximum data pulling resolution no less than 15 minutes |
| Custom Report Builder | User-configurable reports with filters (provider, region, time, KPI) |
| Scheduled Reporting | Automatic generation of periodic reports (e.g., monthly QoS summary) |

## 3.9 Security, Privacy and Compliance

Role-based access control and user management.

Secure authentication (multi-factor if required).

Encryption of data at rest and in transit (TLS).

Logging and audit trails for all critical actions and data access.

Compliance with:

– Relevant national cyber security and data protection regulations.

– BTRC's internal IT/security policies.

## 3.10 System Architecture, Hosting and Performance

Design scalable, modular and fault-tolerant architecture.

Proposal for hosting:

– Government/BTRC/BCC data center, or

– Other arrangement as decided by BTRC.

Performance & availability targets for the platform (e.g. 99.5% uptime).

Backup, disaster recovery and business continuity plans.

### 3.10.1 Minimum Infrastructure Specifications

The platform shall be deployed on infrastructure meeting or exceeding these specifications to ensure adequate performance, availability, and scalability.

Hyper-Converged Infrastructure (HCI):

The system shall be deployed on a Hyper-Converged Infrastructure with the following minimum specifications:

Cluster Configuration:

Specification: Minimum Requirement

Architecture: Hyper-Converged Infrastructure (HCI)

Cluster Size: 5 nodes minimum

Per-Node Specifications:

| Component | Minimum Requirement |
| --- | --- |
| CPU | 32 cores per node |
| Memory | 256 GB RAM per node |
| Boot Drives | 2x per node (mirrored) |
| Data Drives | 3x per node minimum |
| Cache Drives | 2x per node |
| Network Interfaces | 2x 10G SFP+ per node |
| Management Port | Out-of-band management (IPMI/iLO/iDRAC or equivalent) |
| Power Supply | Redundant PSU (1+1 minimum) |

Aggregate Cluster Resources (5 Nodes):

| Resource | Total Minimum |
| --- | --- |
| CPU Cores | 160 cores |
| Memory | 1,280 GB (1.25 TB) |
| Data Drives | 15x |
| Cache Drives | 10x |
| 10G Network Ports | 10x |

Network Fabric:

| Component | Requirement |
| --- | --- |
| Switch Configuration | 1+1 stack configuration |
| Redundancy Protocol | MC-LAG (Multi-Chassis Link Aggregation) support |
| Compatibility | Must be compatible with proposed HCI solution |
| Port Capacity | Sufficient 10G SFP+ ports for all HCI nodes plus growth |

Load Balancer:

| Component | Requirement |
| --- | --- |
| Type | Virtual appliance |
| Deployment | High availability configuration recommended |
| Capabilities | Application load balancing for platform services |

Infrastructure Proposal Requirements:

Vendors shall include in their infrastructure proposal:

HCI Solution: Specific platform and model meeting above specifications

Bill of Materials: Detailed BoM for all hardware components

Network Fabric: Switch model and configuration meeting requirements

Load Balancer: Virtual appliance solution meeting requirements

Compatibility: Certification or documentation confirming component compatibility

Scalability: Path for future node expansion if required

Support: Hardware support and warranty terms

Vendors may propose specifications exceeding these minimums where beneficial for platform performance and reliability.

## 3.11 Testing and Quality Assurance

The Vendor shall prepare and implement a comprehensive Test Plan, including at least:

Unit Testing

Integration Testing

Functional and Non-functional Testing

Performance and Stress Testing

Security and Vulnerability Testing

Compatibility Testing (browsers, mobile devices)

User Acceptance Testing (UAT) with BTRC and selected providers

Test cases, results and defect logs must be documented and submitted to BTRC.

## 3.12 Training and Knowledge Transfer

Develop and provide:

– User Manuals (Admin, Operator, Viewer).

– Technical Documentation (architecture, APIs, data models, deployment scripts).

Conduct structured training sessions for:

– BTRC technical and operational staff.

– Helpdesk/support staff.

Training may include:

– Classroom/virtual sessions.

– Hands-on exercises.

– Evaluation/feedback.

## 3.13 Post-Implementation Support and Maintenance

Provide maintenance and technical support for a defined period (see Section 7), including:

– Bug fixing and minor enhancements.

– Performance tuning.

– Security patching.

– Assistance with onboarding new providers or integrations.

Provide Service Level Agreement (SLA) for:

– Incident response time.

– Resolution time based on severity.

– System availability.